Purpose:

This SOP outlines the procedure for verifying air flow in a meat plant producing products to ensure compliance with food safety standards and to prevent contamination.

Scope:

This procedure applies to all personnel responsible for monitoring and maintaining air flow in production areas.

Responsibilities:

- Quality Assurance (QA) team: Conduct routine air flow verification, document findings, and initiate corrective actions when required.

- Maintenance team: Address any mechanical or structural issues affecting air flow.

- Plant Manager: Ensure proper training and adherence to this SOP.

Materials Needed:

- Candle or lighter

- Tissue paper strips (2-3 inches wide)

- Clipboard, pen, and air flow verification log sheet

- HVAC system maintenance records

Procedure:

- Preparation:

  - Ensure the production area is operational and that all doors are in their normal state (open or closed) as per standard conditions.

  - Verify that HVAC systems are functioning.

  - Gather necessary materials.

- Air Flow Verification Using Tissue Paper:

  - Attach a strip of tissue paper to a small stick or hold it with your hand.

  - Position the tissue paper near the same critical air flow points.

  - Observe the movement of the tissue:

    - Tissue moving inward toward the production/processing  area confirms positive air flow.

    - Tissue moving outward or remaining static indicates potential air flow issues.

- Documentation:

  - Record observations on the air flow verification log sheet, noting the date, time, location, and results.

  - Mark areas with improper air flow for immediate corrective action.

- Corrective Actions:

  - If positive air flow is not verified:

    - Check for obstructions in air ducts or vents.

    - Inspect door seals and ensure they close properly.

    - Notify the maintenance team to inspect and repair HVAC systems as needed.

    - Increase cleaning and sanitation frequency in affected areas until the issue is resolved.

  - Conduct a re-test after corrective actions have been implemented and document results.

- Frequency:

  - Conduct air flow verification annually or when changes are made to the building design, construction, or operation of the facility.

- Training:

  - All personnel responsible for air flow verification must be trained on this SOP annually and whenever updates are made.

Records

Air Flow Verification Log